The 2007 Kubota L3400 is a compact utility tractor known for its versatility and reliability. It features a 3-cylinder, 1.6L diesel engine producing 34.7 horsepower. The L3400 comes in both 2WD and 4WD configurations, with various transmission options including gear and hydrostatic (HST). It's part of the Standard L Series and was produced from 2004 to 2010.

Here's a more detailed breakdown of the specs:

Engine:

- Engine: 3-cylinder Kubota diesel.

- Horsepower: 34.7 hp.

- Displacement: 1.6L (100.5 cu in).

- Aspiration: E-TVCS indirect injection.

- PTO Power: 29 hp.

- Engine Model: L3400 Gear.

Transmission:

- Type: Sliding gear constant mesh or hydrostatic (HST).

- Gear Options: 8 forward and 4 reverse gears (for gear models), or infinite (3-range) forward and reverse (for HST).

- Max Speed: 13.2 mph.

Dimensions:

- Length: 110.6 inches (280 cm).

- Width: 51.4 inches (130 cm).

- Height: 85 inches (215 cm).

- Wheelbase: 63.3 inches (160 cm).

- Ground Clearance: 13.4 inches (34 cm).

Other Key Specs:

- Operating Weight: 2,403 lbs (1090 kg) (2WD gear) to 2,600 lbs (1179 kg) (4WD hydro).

- Fuel Capacity: 9 gallons.

- 3 Point Hitch Lift Capacity (at 24 inches): 1,435.3 lbs.

- Turning Radius (2WD/4WD): 7.9 ft (2.4 m) / 8.2 ft (2.5 m).

- Tire Sizes: (Front) 5-15 / (Rear) 11.2-24 (2WD/4WD).

Other Information:

- The L3400 is known for its user-friendly features like a spacious semi-flat deck and convenient control levers.

- It's a popular choice for compact farm and landscaping work.

- Kubota tractors are generally known for their longevity, with many well-maintained machines lasting 4,500 to 5,500 hours.